Rapid RADAR Plotting Packet

Nav 315 – Intro to Integrated Bridges

Created by: Mate Tamera Gilmartin

(CC) 2020

Table of Contents

Rules for RADAR

Part 1: One Target Plots (Page 8)

Relative Motion Triangle - Instructions

Plots 1-5 (Find: RM, TM, CPA, TCPA) – Homework

Plots 1-5 (Find: RM, TM, CPA, TCPA) - Answers

Unique Cases - Instructions

Plots 6-10 (Find: RM, TM, CPA, TCPA) – Homework

Plots 6-10 (Find: RM, TM, CPA, TCPA) – Answers

Course Change & Speed Change for 1 Target – Instructions

Plots 1-5 (Find: Situation, Giveway/Standon, New Course, New Speed) – Homework

Plots 1-5 (Find: Situation, Giveway/Standon, New Course, New Speed) – Answers

Plots 6-10 (Find: Situation, Giveway/Standon, New Course, New Speed) – Homework

Plots 6-10 (Find: Situation, Giveway/Standon, New Course, New Speed) – Homework

Part 2: Multiple Target Plots (Page 38)

Course Change & Speed Change for Multiple Targets – Instructions

Plots 11-15 (2 Targets)

Plots 16-20 (3 Targets)

Part 3: Calculator-less Calculations (Page 54)

Tenth’s Rule

Logarithmic Time, Speed, and Distance Scale

Calculator-less Calcs – Homework

Calculator-less Calcs - Answers

Repeat Plots 16-20 without a calculator – Homework

Repeat Plots 16-20 without a calculator -Answers

Rules for RADAR

In Collision Avoidance, we will be utilizing the rules to make appropriate decisions on what action to take when interacting with a dangerous target, or a target with a Risk of Collision. In the Rules of the Road class, we will discuss the intricacy of the Rules so that you understand them fully. Here is a reduced version of the Rules, which simplifies them in regards to how they are interpreted in a Collision Avoidance type situation with the RADAR.

Anytime Rules (These rules must always be adhered to in any condition)

- Maintain a proper look out utilizing all available means: this means you should use everything to determine if there is anyone out there which could be an issue, including the RADAR (Rule 5)

- Maintain a Safe Speed, this means travel only so fast as is appropriate. You must be able to avoid the other vessel, if you can’t, you were going too fast: Keep in mind the visibility, the maneuverability of the vessel, how many targets are out there, how close you are to land, weather conditions, and the effect which draft may have on the maneuverability of the vessel. It also specifically mentions the RADAR and states to keep in mind the characteristics and constraints of the RADAR. If you are on a small scale, if your picture is distorted, if you have potential small vessels, ice or other objects which might be missed, you should slow down. It also mentions that you should use the RADAR to try and determine the visibility utilizing the RADAR which is done by determining the point at which an object just disappears and checking its range on the RADAR, this is how far you can see, or your visibility.  (Rule 6)

- Everyone must try to determine if there is a Risk of Collision using all resources available (this includes sound signals). Also if you are in doubt, assume the worst, and assume that there is a Risk of Collision and that you should do something. It mentions to use the RADAR (as it is included in all means available) and that you should use long range scanning (if you are on a smaller scale, punch out your scale and look at a larger zoom just in case there is someone sneaking up on the horizon). It mentions to begin plotting the object to determine the relative motion of the object, letting you know if there is a Risk of Collision as soon as possible. It also mentions that you should not trust “scanty information”. This means partial information or that the information is not complete. (Rule 7)

  - Risk of Collision is defined as being “Steady bearing, Decreasing Range”.

  - It mentions that even if there is a bearing change, sometimes a Risk of Collision still exists. This is true when dealing with an extremely large tow, as well as heavy currents

- If you take action to avoid collision it must be: (Rule 8)

  - Positive (make the situation better)

  - Early (don’t wait to the last minute)

  - One large change (a bunch of small changes can be confusing or not seen by the other vessel)

  - Usually a course change fixes most situations, but sometimes you might have to do a speed change, or a speed change AND a course change.

  - Make sure what you do doesn’t cause another situation

  - Once action is taken, watch and make sure everything goes as planned. Things can still go wrong even if you took action.

    - Watch until they are “Well past and clear” (this means that no matter what, at this point they cannot hit you)

  - If you need more time to assess the situation, slow down. You may need to stop completely

  - If you are giveway vessel, take action. The other vessel wants to know what you are going to do

  - If you are give way vessel, take an early and large action (Rule 16)

  - If you are the stand on vessel, maintain your course and speed. Unless you think the other vessel isn’t taking appropriate action, then you can take action to avoid. But if it reaches a point where no matter what the other vessel does, you will hit the other vessel, you MUST take action at this point. You probably should have acted earlier, but you really have to do something now or there will be a collision. Avoid turning to port if they are on your port side(Rule 17)

  - If you are the stand on vessel, you are still required to follow all of these rules in regards to taking action if you do take action. This means stand on vessels must take action which is:

    - Positive (make the situation better)

    - Early (don’t wait to the last minute)

    - One large change (a bunch of small changes can be confusing or not seen by the other vessel)

Rules when in Sight of One Another (Can you see the other vessel?)

- Overtaking the other vessel. Risk of Collision and you are going faster than the other vessel on a similar heading. They are ahead of you (you can see the stern light) (Rule 13)

  - Avoid them. You can pass on either side, just don’t hit them

  - If it starts as an overtaking situation, it stays an overtaking situation

Them

US

- Being overtaken. Risk of Collision and you are going slower than the other vessel on a similar heading. They are behind you. (Rule 13)

  - You are stand on. Do not turn into them.

US

Them

- Hierarchy of vessels

  - New Reels Catch Fish So Purchase Some

    - NUC

    - RAM

    - CBD

    - Fishing

    - Sailing

    - PDV

    - Seaplane/Wig

- Head on. Risk of Collision and they are ahead of you on a reciprocal course. (You can see both Side Lights, and the Masthead lights are nearly in a row) (Rule 14)

Them

Both vessels turn to STBD

US

- Crossing with other vessel to port. (you can see their red side light) (Rule 15)

  - Stand on, they must avoid you

Them

US

- Crossing with other vessel to STBD. (you can see their green side light) (Rule 15)

  - Giveway, you must avoid them. Do not pass ahead. Slow down or turn to STBD

Them

US

Restricted Visibility (Rule 19)

There is no special Right of Way when it comes to Restricted Visibility; we are all equally stumbling around in the dark.

Any vessel with a risk of collision with any other vessel shall:

    - If the vessel is ahead – Avoid turning to port

    - If the vessel is abaft the beam – Avoid turning towards them

    - If they don’t show up on the RADAR, and they sound like they are forward of the beam, slow down until you can determine what is going on. You may have to slow to bare steerage way (the slowest you can go and still control your course)

Heading

FWD of Beam

Avoid Turn to Port

Abaft of Beam

Avoid Turn Towards Target

Ok, that still seems like a lot….. so let’s simplify even more for this course. All vessels will be PDV so you will not have to consider Hierarchy.

First ask, can you see the other vessel?

Yes (In sight) – What type of situation is it?

- Overtaking (See their Stern Light): You are Giveway. Avoid them on either side

- Overtaking (They are behind you): You are Standon. Maintain Course and Speed or take action if they are not taking action. Do not alter course to Port for a vessel on your port side.

- Head On (See both Side lights and Masthead lights are in a line): You are Giveway. Turn to STBD

- Crossing (See their STBD Light): You are Giveway. Turn to STBD or slow down (they pass ahead)

- Crossing (See their Port Light): You are Standon. Maintain Course and speed or take action if they are not taking action. Do not alter course to Port for a vessel on your port side

No (Restricted Visibility) – Are they ahead of your beam?

      - Yes – Avoid turn to port: Slow down or alter course to STBD

      - No – Avoid turning towards them

One Last note, Multiple Vessels are not mentioned specifically in the Rules. This makes it a “special circumstance” and would fall under Rule 2. Rule 2 says basically says to do whatever you can to avoid collision, but also take action in accordance with good seamanship. Ultimately, try to follow the rules as closely as you possibly can, but you may have to depart from the rules

- For this class, you will identify the first target to take action for. Treat it as an individual case and solve for this vessel. Then check for the other vessels and see if it fixes the entire situation. If it does not, move to the next vessel and see if taking action for that vessel would fix the entire situation.

- To determine target to take action for identify in this order:

  - Target with the soonest TCPA – You will hit this one first, so take care of it first

  - Target which is furthest to the right – If you are altering course to the right, this will tend to take care of all the others at the same time

  - Target with the smallest CPA

Part 1: One Target Plots

Relative Motion Triangle – Instructions

1. Take a Range and Bearing of a target, plot it on your Maneuvering Board

Label this point “R”

2. Wait some time, usually intervals of 6 minutes as it makes the math much easier (tenths rule)

3. Take a Range and Bearing of the target again and plot it’s new position on the Maneuvering Board

Label this point “M”

4. Connect the two points, this creates your Relative Motion Line “RM”. It shows how the target is moving relative to you. If the line goes through the center (This is you on the RADAR screen), it will hit you.

R to M is RELATIVE Motion

5. Find the point on the line which is closest to the center of the circle. This is most accurately done by using your triangle to find where the point is perpendicular to the RM line. This is the CPA (Closest Point of Approach). The distance from the center to the point you marked as CPA will tell you the closest they will get to you.

5. Next draw your course going backwards from “R”.

6. Using the same time interval for time in-between plots of the target, calculate how far you would have traveled at your current speed. Measure along the course line drawn backwards from “R” and plot point “E”

7. Connect “E” and “M”. This is the True Motion of the other vessel

E to R is OUR “True” Motion

E to M Is THEM “True” Motion

8. Solve for True Motion:

For True Speed, measure the length of the E-M line

Convert this distance to Speed using: S=D/T

(Don’t forget to convert from minutes to hours)

S = 2.3NM / (6min/60)

S = 2.3NM / .1hrs

True Speed = 23kts

For True Course, measure the angle of the E-M line, and then move it to the center. Double check that you are going in the correct direction (E to M is THEM)

True Course: 280°T

9. Solve for Relative Motion, repeat for the R-M line:

For Relative Speed, measure the length of the R-M line

Convert this distance to Speed using: S=D/T

(Don’t forget to convert from minutes to hours)

S = 3.0NM / (6min/60)

S = 3.0NM / .1hrs

Relative Speed = 30kts

For Relative Course, measure the angle of the R-M line, and then move it to the center. Double check that you are going in the correct direction (R to M is Relative Motion)

Relative Course: 228°T

10. Solve for TCPA:

Measure the distance from the first position to the CPA.

Convert this distance to time using: T=D/S

(Be sure you are using the Relative Speed as it is traveling across the RM line)

T = 9.8NM / 30kts

Time = .326 hours

Time = .326 hours x 60 = 19.6 min

Add this time to the time of R

TCPA = 1200 + 19.6 min

TCPA = 1219.6

11. Solve for Situation and if Giveway or Stand-on:

Draw ourselves in the center, draw them next to M based upon E-M

What Rules of the Road situation is it?

Crossing, we are giveway because they

are on our STBD side

Plots 1-5 (Find: RM, TM, CPA, TCPA) – Homework

Plot 1 – One Target Plot

Own Ship Info: 		Course: 000.0°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: 1224

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 074.6°T | 10.3NM |
| 1206 | 074.4°T | 8.9NM |
| 1212 | 074.2°T | 7.5NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plot 2 – One Target Plot

Own Ship Info: 		Course: 34O.0°T		Speed: 8.0kts

Minimum CPA: 1.0NM

MX: Time - 1230

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 169.5°T | 5.8NM |
| 1205 | 169.3°T | 5.2NM |
| 1210 | 169.1°T | 4.5NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plot 3 – One Target Plot

Own Ship Info: 		Course: 202.0°T		Speed: 12.4kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 220.8°T | 12.2NM |
| 1206 | 219.9°T | 10.0NM |
| 1212 | 218.6°T | 7.9NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plot 4 – One Target Plot

Own Ship Info: 		Course: 000.0°T		Speed: 10.0kts

Minimum CPA: 1.0NM

MX: Time - 1220

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 068.1°T | 4.4NM |
| 1205 | 067.7°T | 3.8NM |
| 1210 | 067.0°T | 3.3NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plot 5 – One Target Plot

Own Ship Info: 		Course: 060.0°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 085.0°T | 8.9NM |
| 1206 | 083.6°T | 7.2NM |
| 1212 | 081.3°T | 5.5NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plots 1-5 (Find: RM, TM, CPA, TCPA) – Answers

Plot 1 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 255.0°T | 12.3Kkts | 297.0°T | 15.0kts | 0.2NM | 1245 | Crossing | Giveway |

Plot 2 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 351.0°T | 7.5kts | 345.0°T | 15.0 | 0.2NM | 1249 | Overtaking | Stand-on |

Plot 3 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 044.0°T | 22.0kts | 068.5°T | 12.0kts | 0.8NM | 1234 | Crossing | Giveway |

Plot 4 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 256.0°T | 6.5kts | 325.0°T | 10.0kts | 0.6NM | 1238 | Crossing | Giveway |

Plot 5 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 271.0°T | 16.0kts | 303.0°T | 9.5kts | 0.9NM | 1234 | Crossing | Giveway |

Unique Cases – Instructions

Sometimes, the Relative Motion Triangle “collapses” and you are left with a line. This is usually one of the special cases which include “Same Course/Same Speed”, “Opposite Course/Same Speed”, “Target is DIW”. DIW is Dead-In-Water or has no True Motion.

No E-M     Them Motion

R & M

E

E

R

M

E-R OUR Motion

E-M Them Motion

E-M Them Motion

E-R OUR Motion

E-R OUR Motion

E & M

R

Opposite Course/Same Speed

DIW

Same Course/Same Speed

Sometimes, the dots do not align perfectly such as these cases, but the triangle has collapsed. Best thing to do in these cases is to draw your motion lines (E to M is Them) (E to R is Our) (RM is Relative Motion), and then measure the direct values for those lines/motion vectors.

Plots 6-10 (Find: RM, TM, CPA, TCPA) – Homework

Plot 6 – One Target Plot

Own Ship Info: 		Course: 000.0°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 000.0°T | 11.1NM |
| 1206 | 000.0°T | 9.2NM |
| 1212 | 000.0°T | 7.2NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plot 7 – One Target Plot

Own Ship Info: 		Course: 000.0°T		Speed: 10.0kts

Minimum CPA: 1.0NM

MX: Time - 1220

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 180.0°T | 4.9NM |
| 1205 | 180.0°T | 4.9NM |
| 1210 | 180.0°T | 4.9NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plot 8 – One Target Plot

Own Ship Info: 		Course: 234.0°T		Speed: 8.7kts

Minimum CPA: 2.0NM

MX: Time - 1220

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 239.3°T | 11.7NM |
| 1205 | 240.1°T | 10.2NM |
| 1210 | 241.2°T | 78.6NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plot 9 – One Target Plot

Own Ship Info: 		Course: 150.0°T		Speed: 12.4kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 158.9°T | 8.7NM |
| 1206 | 160.4°T | 7.5NM |
| 1212 | 162.4°T | 6.3NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plot 10 – One Target Plot

Own Ship Info: 		Course: 275.0°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 281.8°T | 9.8NM |
| 1206 | 283.5°T | 7.8NM |
| 1212 | 286.4°T | 5.8NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |

Plots 6-10 (Find: RM, TM, CPA, TCPA) – Answers

Plot 6 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 180.0°T | 20.0kts | 180.0°T | 10kts | 0.0NM | 1234 | Head-on | Giveway |

Plot 7 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N/A | N/A | 000.0°T | 10.0kts | N/A | N/A | Same Course Same Speed | No Risk of Collision |

Plot 8 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 054.0°T | 19.0kts | 056.0°T | 11.0kts | 01.1NM | 1238 | Head-on | Giveway |

Plot 9 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 330.0°T | 12.3kts | DIW | DIW | 1.4NM | 1241 | DIW | Giveway |

Plot 10 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 095.0°T | 19.5kts | 095.0°T | 10.0kts | 1.2NM | 1230 | Head-on | Giveway |

Relative Motion Triangle – Instructions

Course Change and Speed Change

Although we are determining whether you are Stand-on or Giveway, we will be maneuvering for all targets to make sure that we have an appropriate CPA. We are taking action under the second stage of Stand-On which states, “You may take action if the other target is not taking action”.

1. We now need to find Mx (Maneuvering point). There are many ways to do this. It is sometimes based on a distance (ie. We will be maneuvering when the target is 5NM from us) or based on a time (ie. We will be maneuvering when the time is 1218 or we will be maneuvering 6 min after M). For this packet, we will be using a time to maneuver because this is more practical than waiting for an object to be a certain distance away. We will also be basing it off of the previous time intervals to make it a simple DR.

For example, R was at 0000 and M was at 0006. Mx will be at 0012.

This will be an easy 6min DR.

Since the distance from R to M was 6 minutes, we can just measure this length.

If you are doing a time which was not used in the triangle, you will need to calculate the distance you will travel from M until Mx, and then plot that distance from M.

2. Now we need to put in our minimum CPA circle. For this example, the Min CPA is 2.0NM which also aligns with the 2.0 ring. If it does not line up on a ring, you must draw this circle in with a compass.

Here is our Min CPA circle at 2.0NM

3. Then we determine the direction and amount that we need the RM line to bend at Mx. First draw the direction we want to take action, in this case to STBD

Notice the RM line goes ahead and to port of our vessel if we turn to STBD. When we begin trying options, make sure that the new RM line will do the same thing.

4. Put your triangle on Mx and move it so that it just barely touches the Min CPA. Also make sure it goes ahead and down our port side, similar to our drawing of the action of turning to STBD. Draw this line in.

5. Shift this new RM line from Mx, back to M. “Move Mx to M”

Draw backwards through M

6. For a Course Change:

The length of E-R represents the speed of the vessel. So, if we want to keep the same speed, lets just measure the length of this line.

Now “Swing” R to the right so that it is on the new R-M line. Make sure that E does not move. We cannot move E or M because that would be changing THEM.

This is labeled R’ because it is the New R. Connect R’ and E and this gives you OUR New Course.

Measure this angle of E-R’ for OUR New Course.

The new course here is 021°T

7. For a Speed Change:

The Length of E-R represents the speed, the angle of E-R represents the course.

If we are changing the speed but not the course, we will leave the direction of the E-R line the same, and just “Shift” R.

Everything is the same up until Step 6, but this time, move R down to the intersection of E-R and R-M.

Measure the length of E-R’ and calculate for speed.

S = 1.8NM / (6min/60)

S = 1.8NM / .1hrs

Our New Speed = 18kts

Plots 1-5 (Find: Add New Course, New Speed) – Homework

Plot 1 – One Target Plot

Own Ship Info: 		Course: 000.0°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: 1224

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 074.6°T | 10.3NM |
| 1206 | 074.4°T | 8.9NM |
| 1212 | 074.2°T | 7.5NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plot 2 – One Target Plot

Own Ship Info: 		Course: 34O.0°T		Speed: 8.0kts

Minimum CPA: 1.0NM

MX: Time - 1230

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 169.5°T | 5.8NM |
| 1205 | 169.3°T | 5.2NM |
| 1210 | 169.1°T | 4.5NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plot 3 – One Target Plot

Own Ship Info: 		Course: 202.0°T		Speed: 12.4kts

Minimum CPA: 3.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 220.8°T | 12.2NM |
| 1206 | 219.9°T | 10.0NM |
| 1212 | 218.6°T | 7.9NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plot 4 – One Target Plot

Own Ship Info: 		Course: 000.0°T		Speed: 10.0kts

Minimum CPA: 1.0NM

MX: Time - 1220

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 068.1°T | 4.4NM |
| 1205 | 067.7°T | 3.8NM |
| 1210 | 067.0°T | 3.3NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plot 5 – One Target Plot

Own Ship Info: 		Course: 060.0°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 085.0°T | 8.9NM |
| 1206 | 083.6°T | 7.2NM |
| 1212 | 081.3°T | 5.5NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plots 1-5 (Find: Add New Course, New Speed) – Answers

Plot 1 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 255.0°T | 12.3Kkts | 297.0°T | 15.0kts | 0.2NM | 1245 | Crossing | Giveway | 071°T | 5kts |

Plot 2 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 351.0°T | 7.5kts | 345.0°T | 15.0 | 0.2NM | 1249 | Overtaking | Stand-on | 202°T | NA |

Plot 3 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 044.0°T | 22.0kts | 068.5°T | 12.0kts | 0.8NM | 1234 | Crossing | Giveway | 227°T | 4kts |

Plot 4 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256.0°T | 6.5kts | 325.0°T | 10.0kts | 0.6NM | 1238 | Crossing | Giveway | 028°T | 7kts |

Plot 5 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 091.0°T | 16.0kts | 303.0°T | 9.5kts | 0.9NM | 1234 | Crossing | Giveway | 094°T | 3.5kts |

Plots 6-10 (Add New Course, New Speed) – Homework

Plot 6 – One Target Plot

Own Ship Info: 		Course: 000.0°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 000.0°T | 11.1NM |
| 1206 | 000.0°T | 9.2NM |
| 1212 | 000.0°T | 7.2NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plot 7 – One Target Plot

Own Ship Info: 		Course: 000.0°T		Speed: 10.0kts

Minimum CPA: 1.0NM

MX: Time - 1220

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 180.0°T | 4.9NM |
| 1205 | 180.0°T | 4.9NM |
| 1210 | 180.0°T | 4.9NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plot 8 – One Target Plot

Own Ship Info: 		Course: 234.0°T		Speed: 8.7kts

Minimum CPA: 2.0NM

MX: Time - 1220

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 239.3°T | 11.7NM |
| 1205 | 240.1°T | 10.2NM |
| 1210 | 241.2°T | 78.6NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plot 9 – One Target Plot

Own Ship Info: 		Course: 150.0°T		Speed: 12.4kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 158.9°T | 8.7NM |
| 1206 | 160.4°T | 7.5NM |
| 1212 | 162.4°T | 6.3NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plot 10 – One Target Plot

Own Ship Info: 		Course: 275.0°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | Bearing | Range |
| --- | --- | --- |
| 1200 | 281.8°T | 9.8NM |
| 1206 | 283.5°T | 7.8NM |
| 1212 | 286.4°T | 5.8NM |

Other Ship Info:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Plots 6-10 (Find: Add New Course, New Speed) – Answers

Plot 6 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 180.0°T | 20.0kts | 180.0°T | 10kts | 0.0NM | 1234 | Head-on | Giveway | 046°T | NA |

Plot 7 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N/A | N/A | 000.0°T | 10.0kts | N/A | N/A | Same Course Same Speed | No Risk of Collision | NA | NA |

Since there is no Risk of Collision, it does not have a situation and you do not have to maneuver for this target.

Plot 8 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 054.0°T | 19.0kts | 056.0°T | 11.0kts | 01.1NM | 1238 | Head-on | Giveway | 305°T | NA |

Plot 9 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 330.0°T | 12.3kts | DIW | DIW | 1.4NM | 1241 | DIW | Giveway | 187°T | NA |

Plot 10 – One Target Plot:

| Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 095.0°T | 19.5kts | 095.0°T | 10.0kts | 1.2NM | 1230 | Head-on | Giveway | 324°T | NA |

Part 2: Multiple Targets Plots

Multiple Target Plot – Instructions

Course Change and Speed Change

For Multiple Target plots, focus on each target individually up until the point where you determine course change. Yes, you are doing all three at the same time, but they are not dependent upon each other, so it is easier to focus on each one separately.

You will basically end up with several plots all on the same page. Two targets or three targets are the same, you just have more targets on the same page, so we will focus on two targets in this example.

1. First, we must decide which target to maneuver for. We often call this the “Dangerous Target” or “Maneuvering Target”. To decide this, we ask ourselves the following questions in order:

1. Who will we “hit” first? Who has the earliest TCPA

In this example, the TCPA is the same, it is 0020 for both. If the TCPAs are the same, or fairly close, you move on to the next question.

2. Who is the furthest right?

We ask this because generally, we will be altering course to STBD. When altering course to STBD for the right most vessel, it tends to help the situation for the other vessels as well.

In this case, Target 2 is the furthest right. We will be maneuvering for Target 2.

Although we are determining whether you are Stand-on or Giveway, we will be maneuving for all targets to make sure that we have an appropriate CPA. We are taking action under the second stage of Stand-On which states, “You may take action if the other target is not taking action”.

2. Next, we will focus on the “Dangerous Target” and solve a Course Change for this target. This follows the same steps we had before.

Find Mx for Dangerous Target. In this example, we used 0012

Draw New Relative Motion Line from Mx so that it is touching the Minimum CPA.

In this case we used 2.0NM for the Min CPA.

Also, make sure the direction of the new Relative Motion Line is correct. Here we drew us altering to STBD, which had the RM line going Ahead and Down our Port Side. Make sure the New RM line is doing the Same thing.

Shift New RM from Mx to M. Draw backwards from M

“Swing” R to find R’

3. Now, we must double check that this also fixed the minimum CPA issue we had with Target 1.

Measure OUR New Course

Shift to E on Target 1 and draw OUR New Course

Measure the length of E-R. This represents OUR Speed. Swing R to OUR New Course.

Measure the New Relative Motion line on Target 1 and shift it to the Mx point on Target 1. Draw the New Relative Motion line at Mx for Target 1.

Ask yourself if this New Relative Motion line for Target 1 is also good, or if it causes more issues. This one looks good to me!

**If it does not work, you need to repeat Step 3, but for the target where it still has a Risk of Collision.**

To do this, erase everything from Step 3. Shift your focus to the new “dangerous target” and repeat Step 3 for the new “dangerous target”.

Plots 11-15 (2 Target Plots) – Homework

Plot 11 – Two Target Plot

Own Ship Info: 		Course: 000°T		Speed: 10.0kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range |
| --- | --- | --- | --- | --- |
| 1200 | 070.1°T | 8.4NM | 359.9°T | 12.1NM |
| 1206 | 069.8°T | 6.9NM | 359.9°T | 9.9NM |
| 1212 | 069.2°T | 5.4NM | 359.9°T | 7.8NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |

Plot 12 – Two Target Plot

Own Ship Info: 		Course: 000°T		Speed: 8.7kts

Minimum CPA: 1.5NM

MX: Time - 1220

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range |
| --- | --- | --- | --- | --- |
| 1200 | 071.6°T | 5.9NM | 238.0°T | 6.0NM |
| 1206 | 070.5°T | 5.1NM | 239.0°T | 5.1NM |
| 1212 | 069.0°T | 4.4NM | 240.5°T | 4.3NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |

Plot 13 – Two Target Plot

Own Ship Info: 		Course: 020°T		Speed: 10kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range |
| --- | --- | --- | --- | --- |
| 1200 | 079.1°T | 9.5NM | 343.8°T | 6.9NM |
| 1206 | 079.1°T | 8.3NM | 343.5°T | 5.7NM |
| 1212 | 079.1°T | 7.1NM | 343.1°T | 4.6NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |

Plot 14 – Two Target Plot

Own Ship Info: 		Course: 042°T		Speed: 11.2kts

Minimum CPA: 1.5NM

MX: Time - 1209

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range |
| --- | --- | --- | --- | --- |
| 1200 | 045.1°T | 5.4NM | 347.3°T | 6.1NM |
| 1206 | 045.0°T | 3.5NM | 346.0°T | 5.1NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |

Plot 15 – Two Target Plot

Own Ship Info: 		Course: 275°T		Speed: 10kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range |
| --- | --- | --- | --- | --- |
| 1200 | 275.3°T | 9.7NM | 236.8°T | 10.8NM |
| 1206 | 275.3°T | 7.7NM | 237.1°T | 8.5NM |
| 1212 | 275.3°T | 5.7NM | 239.1°T | 6.3NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |

Plots 11-15 (2 Target Plots) – Answers

Plot 11 – Two Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 254°T | 15kts | 293°T | 15.0kts | 0.1NM | 1233 | Crossing | Giveway | 078°T | 3.5kts |
| T2 | 180°T | 22kts | 180°T | 11.0kts | 0.0NM | 1233 | Head-on | Giveway | - | - |

Plot 12 – Two Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 260.0°T | 9kts | 311°T | 12.0kts | 0.9NM | 1234 | Overtaking | Stand-On | 044°T | 7.5kts |
| T2 | 053.0°T | 10kts | 027°T | 17.0kts | 0.5NM | 1239 | Crossing | Giveway | - | - |

Plot 13 – Two Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 260.0°T | 12kts | 313°T | 10kts | 0.2NM | 1248 | Crossing | Giveway | 062°T | 6.5kts |
| T2 | 166.0°T | 12 | 104°T | 6.5kts | 0.2NM | 1234 | Crossing | Stand-on | - | - |

Check 2 First because of earlier TCPA, but it does not provide enough of a correction for T1 and is still a dangerous situation. You must then redo the calculation base

Plot 14 – Two Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 229.0°T | 18.5kts | 229.0°T | 7.0kts | 0.0NM | 1218 | Head-On | Giveway | 104.5°T | NA |
| T2 | 175.0°T | 10kts | 103.5°T | 8.5kts | 0.8NM | 1237 | Crossing | Stand-on | - | - |

Plot 15 – Two Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 095.5°T | 20kts | 068°T | 21kts | 0.0NM | 1230 | Crossing | Stand-on | 076°T | NA |
| T2 | 054.0°T | 23kts | 037°T | 29kts | 0.5NM | 1228 | Overtaking | Stand-on | - | - |

Although we are stand-on, we decide to take action as the other vessel is not taking action

Plots 16-20 (3 Target Plots) – Homework

Plot 16 – Three Target Plot

Own Ship Info: 		Course: 000°T		Speed: 10kts

Minimum CPA: 1.0NM

MX: Time - 1212

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range | T3 Bearing | T3 Range |
| --- | --- | --- | --- | --- | --- | --- |
| 1200 | 325.9°T | 5.1NM | 358.5°T | 5.0NM | 075.9°T | 6.5NM |
| 1206 | 318°T | 4.3NM | 358.5°T | 5.0NM | 076.1°T | 4.9NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |

Plot 17 – Three Target Plot

Own Ship Info: 		Course: 025°T		Speed: 10kts

Minimum CPA: 2.0NM

MX: Time - 1218

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range | T3 Bearing | T3 Range |
| --- | --- | --- | --- | --- | --- | --- |
| 1200 | 252°T | 9.5NM | 025.1°T | 7.8NM | 048.1°T | 9.9NM |
| 1206 | 252°T | 9.5NM | 025.1°T | 6.8NM | 047.9°T | 8.3NM |
| 1212 | 252°T | 9.5NM | 025.1°T | 5.8NM | 047.7°T | 6.6NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |

Plot 18 – Three Target Plot

Own Ship Info: 		Course: 310°T		Speed: 10kts

Minimum CPA: 1.0NM

MX: Time - 1224

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range | T3 Bearing | T3 Range |
| --- | --- | --- | --- | --- | --- | --- |
| 1200 | 246.7°T | 6.6NM | 310.4°T | 6.5NM | 003.1°T | 5.6NM |
| 1206 | 247.2°T | 5.7NM | 310.4°T | 5.6NM | 003.1°T | 4.5NM |
| 1212 | 248.0°T | 4.9NM | 310.4°T | 4.6NM | 003.1°T | 3.5NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |

Plot 19 – Three Target Plot

Own Ship Info: 		Course: 270°T		Speed: 8.7kts

Minimum CPA: 1.0NM

MX: Time - 1224

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range | T3 Bearing | T3 Range |
| --- | --- | --- | --- | --- | --- | --- |
| 1200 | 234.8°T | 6.1NM | 270.7°T | 6.2NM | 323.5°T | 6.5NM |
| 1206 | 234.8°T | 6.1NM | 270.7°T | 5.4NM | 323.0°T | 5.5NM |
| 1212 | 234.8°T | 6.1NM | 270.7°T | 4.5NM | 322.5°T | 4.5NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |

Plot 20 – Three Target Plot

Own Ship Info: 		Course: 165°T		Speed: 11.2kts

Minimum CPA: 2.0NM

MX: Time - 1212

| Time | T1 Bearing | T1 Range | T2 Bearing | T2 Range | T3 Bearing | T3 Range |
| --- | --- | --- | --- | --- | --- | --- |
| 1200 | 160.1°T | 7.7NM | 176.3°T | 9.8NM | 243.3°T | 8.2NM |
| 1206 | 159.2°T | 6.6NM | 177.4°T | 7.7NM | 244.0°T | 6.6NM |

Other Ship Info:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |

Plots 16-20 (3 Target Plots) – Answers

Plot 16 – Three Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 180°T | 10kts | DIW | DIW | 2.9NM | 1225 | DIW | Giveway | - | - |
| T2 | NA | NA | 000 | 10kts | @CPA | @CPA | Same Course/ Speed | Giveway | - | - |
| T3 | 256°T | 16.5kts | 289°T | 16.5kts | 0.2NM | 1224 | Crossing | Giveway | 065°T | 5kts |

Plot 17 – Three Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | NA | NA | 025°T | 10kts | @ CPA | @ CPA | Same Course/ Speed | Giveway | - | - |
| T2 | 205°T | 10kts | DIW | DIW | 0.0 | 1246 | DIW | Giveway | - | - |
| T3 | 230°T | 16kts | 255°T | 8.9kts | 0.1NM | 1234 | Crossing | Giveway | 062°T | 5kts |

Plot 18 – Three Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 062°T | 12.5kts | 000°T | 10kts | 0.2NM | 1246 | Crossing | Stand-on | - | - |
| T2 | 130°T | 10kts | DIW | DIW | 0.0NM | 1239 | DIW | Giveway | - | - |
| T3 | 184°T | 11kts | 245°T | 8.9kts | 0.1NM | 1230 | Crossing | Giveway | 029°T | 2.5kts |

Plot 19 – Three Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | NA | NA | 270°T | 8.7kts | @ CPA | @CPA | Same Course/ Speed | NA | - | - |
| T2 | 090°T | 8.7 | DIW | DIW | 0.0 | 1243 | DIW | Giveway | - | - |
| T3 | 146°T | 10kts | 200°T | 8.9kts | 0.3NM | 1238 | Crossing | Giveway | 304°T | 6kts |

Plot 20 – Three Target Plot:

| Target | Relative Course | Relative Speed | True Course | True Speed | CPA | TCPA | Situation | Giveway or Stand-on? | New Course | New Speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 344°T | 11kts | DIW | DIW | 0.5NM | 1241 | Head-on | Giveway | - | - |
| T2 | 352°T | 21 | 004°T | 10kts | 0.7NM | 1229 | Head-on | Giveway | - | - |
| T3 | 062°T | 16 | 105°T | 16.0kts | 0.3Nm | 1231 | Crossing | Giveway | 244°T | 5.0kts |

Triangle is small, could be a little inaccurate or has more error due to the small triangle

Part 3: Calculator-less Calculations

Tenth’s Rule

We generally use 6 min intervals in most of our navigation calculations because it is 1/10 of an hour. We do this because it makes the math easy and can do it quickly in our head. This means we can focus on more important things such as not colliding with a target.

To convert Speed to a Distance using a 6 min interval you simply move the decimal back one place. (The number should get smaller)

For example:

If we are going 20kts, we will go 2.0NM in 6 min

D = S x T

D = 20kts x 6min

D = 20kts x .1 hr

D = 2.0

To convert a Distance to a Speed, move the decimal forward one place. (The number should get bigger)

For example:

If we went 1.1NM in 6 min, we were going 11kts

S = D /  T

S = 1.1NM / 6min

S = 1.1NM / .1min

S = 11kts

This also works with other multiples of 6min. 12min is .2hr and 3min is .05hr

So, for 12min, double and move the decimal place

For 3min, cut it in half and move the decimal place

We are going 22kts for 3 min			We are going 15kts for 12min

D = 22kts x .05hr				D = 15 x .2

D = 1.1NM					D = 3.0NM

Tenth’s Rule – Homework

Distance and Time

1. 	1NM & 6min        	Speed:  ________

2.	 2.2NM & 6min		Speed:  ________

3. 	3.1NM & 6min		Speed:  ________

4. 	0.5NM & 6min		Speed:  ________

5.	1.2NM & 6min		Speed:  ________

6.	.6NM & 3min		Speed:  ________

7.	1.2NM & 3min		Speed:  ________

8.	2.4NM & 12min		Speed:  ________

9.	4.2NM & 12min		Speed:  ________

10.	3.0NM & 12min		Speed:  ________

Speed and Time

1. 	15kts & 6min		Distance:   ________

2.	20kts & 6min		Distance:   ________

3.	12kts & 6min		Distance:   ________

4.	8kts & 6min		Distance:   ________

5.	32kts & 6min		Distance:   ________

6.	18kts & 3min		Distance:   ________

7.	22kts & 3min		Distance:   ________

8.	8kts & 12min		Distance:   ________

9.	6kts & 12min		Distance:   ________

10.	24kts & 12min		Distance:   ________

Tenth’s Rule – Answers

Distance and Time

1. 	1NM & 6min        	Speed: 10kts

2.	 2.2NM & 6min		Speed: 22kts

3. 	3.1NM & 6min		Speed:  31kts

4. 	0.5NM & 6min		Speed:  5kts

5.	1.2NM & 6min		Speed:  12kts

6.	.6NM & 3min		Speed:  12kts

7.	1.2NM & 3min		Speed:  24kts

8.	2.4NM & 12min		Speed:  12kts

9.	4.2NM & 12min		Speed:  21kts

10.	3.0NM & 12min		Speed: 15kts

Speed and Time

1. 	15kts & 6min		Distance:   1.5NM

2.	20kts & 6min		Distance:   2.0NM

3.	12kts & 6min		Distance:   1.2NM

4.	8kts & 6min		Distance:   0.8NM

5.	32kts & 6min		Distance:   3.2NM

6.	18kts & 3min		Distance:   0.9NM

7.	22kts & 3min		Distance:   1.1NM

8.	8kts & 12min		Distance:   1.6NM

9.	6kts & 12min		Distance:   1.2NM

10.	24kts & 12min		Distance:   4.8NM

Logarithmic Scale

At the bottom of the Rapid RADAR Sheet, there is also a scale which allows you to calculate anything which is outside of the standard 6min very quickly without a calculator. It is a little awkward at first, but once you get the hang of it, it is extremely fast and easy.

There are really only two rules,

Distance on the Left, Time on the Right

And

Speed on the Left, 60 min on the Right

The plot also tries to give you an example at the bottom, but most people don’t understand what they are trying to show and ignore it. But if you understand the process, you really don’t have to remember anything because the sheet gives you that example.

Here is the example right here:

See how the measurement of Distance on the Left and Time on the right is the same as Speed on the left and 60min on the right.

So, what the example is trying to show, is that you need to find which version of the two rules you have. Do you have distance and time? Or do you have Speed? Whichever complete divider rule you have, start with that spread of the dividers, then shift to what you know from the other rule.

If you have Speed and Distance:

Place the right point of your divider on 60,

and then measure out to your speed.

Pick up your dividers, and place the

Left divider on your Distance

Let your right divider fall down on the scale

Where it falls is your time

If you have Distance and Time:

Place the Left point of your divider on

the Distance

Then place the Right divider on your Time

Pick up the dividers and place the Right

Point of your dividers on 60min

Let the left side fall down to your Time

If you have Time and Speed:

Place the right point of your divider on 60,

and then measure out to your speed.

Pick up your dividers, and place the

Right divider on your Time

Let your Left divider fall down on the scale

Where it falls is your Distance

Logarithmic Scale - Homework

Distance			Time			Speed

1.	1NM				6min			____________

2.	0.3NM				2min			____________

3. 	2.5NM				10min			____________

4.	2.7NM				12min			____________

5. 	14NM				50min			____________

6.	2NM				6min			____________

7.	1NM				____________		8kts

8.	1.6NM				____________		20kts

9.	10NM				____________		26kts

10.	9NM				____________		21kts

11.	12NM				____________		14kts

12.	6NM				____________		17kts

13.	____________			10min			15kts

14.	____________			45min			14kts

15.	____________			6min			19kts

16.	____________			30min			18kts

17.	____________			17min			13.5kts

18.	____________			20min			10kts

Logarithmic Scale - Answers

Distance			Time			Speed

1.	1NM				6min			__10kts______

2.	0.3NM				2min			___9kts______

3. 	2.5NM				10min			___15kts_____

4.	2.7NM				12min			___13.5kts____

5. 	14NM				50min			____16.8kts___

6.	2NM				6min			____20kts_____

7.	1NM				____7.5min___		8kts

8.	1.6NM				___4.8min____		20kts

9.	10NM				___23min____		26kts

10.	9NM				___26min____		21kts

11.	12NM				___51min____		14kts

12.	6NM				___21min____		17kts

13.	___2.5NM____			10min			15kts

14.	___10.5NM___			45min			14kts

15.	___1.9NM____			6min			19kts

16.	___9NM______			30min			18kts

17.	___3.8NM____			17min			13.5kts

18.	___3.3NM____			20min			10kts